Suggested Specification for Double Disc Check Valve

Check valve shall be of the double disc, wafer style with torsion spring induced closure.

Valve shall be Wafer style for bolting between ANSI Class 125 flanges. Valves have a Ductile

Iron body (ASTM A-536 65-45-12) to fit inside 125# ANSI bolt circles, a two piece Stainless

Steel disc (ASTM type 304), type 304 Stainless Steel dual shafts, ASTM A313, type 304

Stainless Steel torsion spring and have an integrally molded elastomer seat vulcanized to the

body. Valve for horizontal flow shall be installed with the shafts in vertical position. Double

Disc Check Valve shall be Pratt Series 740A as manufactured by the Henry Pratt Company.